Hessisches Staatstheater Wiesbaden

Technical Rider

Main Theatre Stage

Last updated 05.05.2023

Contact

Hessisches Staatstheater Wiesbaden
Christian-Zais-Str. 3
65189 Wiesbaden

0611 132 1

Brief description/ Stage area

Platforms I-V are 17m wide and 3m deep all around the platform.

Platforms I-III are so-called double-decker platforms; max. lift 4.50m. The clear height inside (visible) is 3.60m, the clear width approx. 12.20m. The floating floor forms the stage floor here. Platforms I-III can only be lowered to -0.40m.

Platforms IV-V are not double-decker platforms, here only the deck moves, the platform floor stays fixed; max. lift 3.0 m. Platforms IV-V can be lowered 1.98 m.

→ The speeds for raising/lowering the platforms are computer-controlled; the noise of this movement can be clearly heard during ‘open’ movement within a scene.

All platforms are equipped with tilt adjusters; the platforms can be tilted to a total tilt of approx. 10% - however, the tilt adjusters are not integrated into the computer control system.

All platforms can be accessed from the side in the understage area.

It is not possible to visually conceal the platforms when they move up/down, the gap through which they move is too small - i.e. technically visible equipment must be accepted.

There is a drop trap system that can be moved under each platform; by opening the top sliders (‘flaps’, see floor plan), the drop trap can be moved between -2.90m and +/-0.0m.

The turntable of the revolving stage is integrated into the backstage carriage. It has a total area of

17.0m x 15.0m and an overall height of 0.40m, and can be lowered to +/-0.0m using all the platforms; not lower.

Using the tilted platforms and lowering the turntable at the same time is not possible.

View of the proscenium arch from the auditorium

View into the auditorium from backstage

Iron curtain

Proscenium bridge

View of the right side of the stage

View of the left side of the stage

Delivery scenery etc. (goods receiving area, loading bay)

Loading bay

1.1 Stage

1.1.1 Orchestra pit

Parapet			Height								0.93 m

Area			Depth to the forestage						4.58 m

Width between the ends of the parapet			16.08 m

Width between the proscenium boxes				12.23 m

Width between the ends of the forestage			13.23 m

2 platforms		Lift; individually adjustable speed				0 to – 2.50 m

Speed								0.20 m/s

Hoists	2 point hoists, can only be operated together; 			
max. load							500 kg each

Please enquire about whether they can be used

1.1.2	Proscenium

Forestage		Height								1.17 m

Area			Depth from forestage to zero line				3.27 m

Depth from parapet to zero line				7.87 m

Width between the ends of the forestage			13.01 m

Width under safety curtain					11.50 m

Prompter’s box 	Width x depth							0.76 x 0.76 m²

(Zero line: rear edge of technical portal)

1.1.3 Portal

Dimensions		Depth from front edge of safety curtain to zero line		1,28 m

Width under safety curtain					11.50 m

Width of technical portal					11.98 m

Height, adjustable					+ 3.30 bis + 8.40 m

Safety curtain/iron curtain with door; decorated with motifs on audience side

Width								13.00 m

Height								9.27 m

Thickness							0.20 m

Door; 2.50 m to left of centre; opens to stage right

Main curtain		Red velvet; can open horizontally or vertically

Zero track		Number of flybars						3

Length								13.00 m

Diameter							76 mm

Load capacity 							650 kg each

Height								0.20 – 22.29 m

Speed, continuously adjustable 				0 – 1.80 m/s

1.1.4 Stage

Area			Width; inner panorama						17.81 m

Depth; technical portal to panorama (MH 47)			17.00 m

Height			End position flyrods						2.65 m

End position overhead lights (without spotlights)		21.20 m

Height under 1st gallery					9.80 m

Backstage		Through access; width x height				18.00 x 9.00 m

Sides of stage		Through access; width x height				12.54 x 8.49 m

Platforms		Double-decker platforms I - III	max. 12 t		-0.40 to +4.50 m

Platforms IV- V			max. 12 t		-1.90 bis +3.00 m

Tilt		continuously adjustable		max. 10 %

Lift speed	continuously adjustable		0 to 0.12 m/s

Drop trap		Can be moved under all platforms	max. 2 t	10 m x 1 m

Lift speed		4 settings			max. 0.12 m/s

Backstage carriage	Overall height 		can be lowered 		0.40 m

Turntable 	diameter			14.00 m

Rotating speed	0° to 360°			ca. 60 s

Side stage carriages	Area				2 units 			17 m x 6 m

(out of order)		Overall height			can be lowered		0.20 m

1.1.5 Overstage machinery

|  | Number | Ø | Length | Load capacity | Lift (m) | Lift speed |
| --- | --- | --- | --- | --- | --- | --- |
| Mechanical hoists (MH) | 40 units | 76 mm | 17 m | 650 kg each | 0.20 - 22.65 | 0 – 1.80 m/s |
| Manual hoists (MANH) | 7 units | 76 mm | 17 m | 300 kg each | 0.20 - 22.65 | 0 – 1.80 m/s |
| Panorama hoists | 4 units | 76 mm | 15 m | 650 kg each | 0.20 - 22.65 | 0 – 1.80 m/s |
| (2 left, 2 right) |  |  |  |  |  |  |
| Point hoists | 46 units | / | / | 350 kg each | 0.20 - 22.65 | 0 – 1.80 m/s |
| Overhead lighting hoists | 5 units | 60 mm | 12 m | 600 kg each | 0.20 - 21.20 | 0 – 0.50 m/s |
| Gallery hoists | 10 units | 48 mm | 2.5m | 120 kg each | / | / |
| (5 left, 5 right) |  |  |  |  |  |  |

1.2 Transport to and from the theatre

Loading bay		Height							1.08 m

Loading bay gate	Narrowest and lowest point; width x height		2.05 x 2.80 m

Loading path		Distance from storage to stage				approx. 75 m

1.3 Lighting

1.3.1 Lighting booth and back-up console

The lighting control system consists of a Transtechnik Prisma NT + Booster (NTX) as the main console and a Transtechnik Prisma NT + Booster (NTX) as a backup console.

1.3.2 Fixed positions

Auditorium:

2x Robert Juliat D’artagnant 2.5kW HMI + Rainbow 8“

2x 2.5 kW Arri Fresnel lens HMI + H&G DarkVader 300 + H&G MagMax 300

15x JB-lighting P18 HC

6x JB-lighting P12 HC

2x 5kW Arri Fresnel halogen

2x 2kW ENI follow spots halogen + Rainbow 8“

1x PANI 6kW stage projector + zoom optics spot + Rainbow 12“ 6x Chauvet Ovation E-910C

2x ENI 1kW profile spots (conductor) 6x 2kW ENI zoom + Rainbow 8“

2x 2.5 kW Altmann follow spot HMI + H&G SH80 + Rainbow 8“ 2x 1.2kW Altmann zoom halogen

Portal bridge:

6x Mac Viper Performance

6x 2kW Strand Alto F

4x 1,2kW Altmann Zoom 2x 500W low-voltage follow spots 4x 2kW Strand Alto PC 6x 2kW ENI profile spots 26°

Overstage lighting bridges (5):

Each equipped with:

6x MacEncore LED wash (PC) warm 5x MacEncore LED Wash (PC) cold 6x 4flg fluorescent lights (RGB+Coolwhite)

Portal towers left and right:

Each equipped with:

2x MacEncore LED wash (PC) cold 1x JB-lighting P18 HC

1x 2,5 kW Arri Fresnel lens HMI + H&G DarkVader 300 + H&G MagMax 300 2x 2kW ADB PC

Gallery left and right:

Each equipped with:

6x 2kW Altmann zoom 6x 2kW ENI zoom

Track/Aisle towers (10):

Each equipped with:

4x 1,2kW Altmann zoom 2x 1kW PAR 64

Track/Aisle towers (12):

Each equipped with:

2x Robe Parfect 100 RGBW 4x 1kW PAR 64

1.3.3 Mobile equipment

Number of devices

8		2.5 kW Arri Fresnel lens HMI + H&G DarkVader 300 + H&G MagMax 300

12		MacViper performance 16bit

12		MacViper wash 16bit

10		Aisle towers/track stands

12		Partowers

4		ETC Fos4 Fluter

14		Startube RGBW

10		LDDE Slim FR1440

6		LDDE Nanopix 1200

16		Swoboda

1.3.4. Colour scrollers

H&G Madmax 300:

Hagenbach & Grill MagMax 300

Pos	LEE	Rosco	%	Set

1	117	66	0%	0

2	116	95	4%	10

3	161	367	8%	20

4	183	69	12%	30

5	HT181	/	16%	40

6	HT119	/	20%	51

7	HT197	/	24%	61

8	200	3220	30%	76

9	201	3202	36%	91

10	202	3204	40%	102

11	203	3208	44%	112

12	130	0	48%	122

13	152	304	52%	132

14	206	3409	56%	142

15	205	3408	60%	153

16	204	3407	64%	163

17	105	21	68%	173

18	106	26	72%	185

19	113	342	76%	193

20	101	312	82%	209

21	138	388	86%	219

22	242	93	90%	229

23	HT124	/	100%	255

Hagenbach & Grill MagVader 350:

HB Arri

X

Pos	LEE	Rosco	%	Set

1	130	0	0%	0

2	144	L144	10%	25

3	201	3202	25%	63

4	200	3220	40%	102

5	197	HT197	50%	127

6	HT119	/	65%	165

7	71	605	80%	204

8	242	93	100%	255

Rainbow 8“ + 12“:

VB

Pos	LEE	Rosco	% 	Set

1	HT124	/	0%	0

2	242	93	4%	10

3	138	388	8%	20

4	117	66	12%	30

5	140	72	16%	40

6	161	367	20%	51

7	132	68	25%	63

8	HT181	/	28%	71

9	HT119	/	34%	86

10	200	3220	36%	92

11	201	3202	41%	104

12	202	3204	44%	113

13	203	3208	50%	127

14	130	0	54%	135

15	152	304	58%	147

16	206	3409	61%	157

17	205	3408	65%	166

18	204	3407	70%	178

19	105	21	74%	188

20	147	317	78%	199

21	101	312	83%	212

22	164	25	87%	223

23	106	26	91%	232

24	113	342	94%	241

25	136	52	100%	255

1.3.5 Distributors and electrical connections in the stage area

Left side		1x 125A CEE direct

6x 32A CEE direct

16x 32A dimmable

40x 10A dimmable

2x 16A switchable

Right side 		1x 125A CEE

6x 32A CEE

16x 32A dimmable

40x 10A dimmable

2x 16A switchable

Portal tower left	16x 32A dimmable

2x 32A CEE direct

8x 16A switchable

Portal tower right	16x 32A dimmable

2x 32A CEE direct

8x 16A switchable

Turntable		6x 32A dimmable or direct

Backstage		2x 125A CEE direct

12x 32A dimmable

12x 10A dimmable

2x 32A CEE switchable

2x 16A 1pol switchable

Wings left		1x 32A 1pol direct

6x 16A dimmable

1x 32A dimmable

Wings right		1x 32A 1pol direct

6x 16A dimmable

1x 32A dimmable

Dome			6x 32A dimmable

1x 32A CEE direct

1x 16A switchable

Upper circle		30x 32A dimmable

2x 32A CEE direct

4x 16A switchable

Middle circle		8x 32A dimmable

2x 32A CEE direct

4x 16A switchable

Dress circle		10x 32A dimmable

2x 32A CEE switchable

4x 16A switchable

Stalls			6x 32A dimmable

2x 32A CEE direct

1x 16A switchable

1.4 Sound

1.4.1 Auditorium mixing desk

Mixer	Stagetec Aurus Platinum

Yamaha CL-5 Yamaha CL-1 Yamaha QL-1

Audio network	Yamaha Dante

Stagetec Nexus, Starrouter

Periphery	Sound systems: SCS, Q-Lab

Abletonlive

DAWs: Steinberg wavelab, Steinberg Nuendo Waves soundgrid

1.4.2	Sound control desk, behind glass

Mixer	Stagetec Aurus Platinum

Periphery	see above periphery “auditorium mixing desk”

1.4.3	Distributor box structure

Stage and auditorium	approx. 30 mic, line, video, LSP, listening channels 230 V light signals

4x network connections

Control console		Stagetec Nexus (digital audio router)

Ghielmetti (analogue audio router) Dante network

1.4.4	Sound system

Portal	Meyer Sound

Galileo

Stage	Kling+Freitag

Syrincs

Mobile systems	3x Yamaha Rio

Mikroport (sound system made by Sennheiser) 10x transmission distance

2x In-ear Sennheiser various speakers mixer consoles (small)

100 Volt	62 amplifier channels on the stage and in the auditorium

1.5 Video

Front projection

Panasonic DZ21K, 20000 lumen, 1920 x 1200 pixel, light + 3 DLP-chip

Back projection

Eiki LC-XT5, 15000 lumen, 1024 x 768 pixel, light + LCD TFT panel

PC

Mac Pro, 32GB, NVIDIA Quadro K5000, OS X El Capitan

Software

Projection control with QLab 4

Laptop for mobile use with QLab 3

Cameras (used in both the small and the main stages)

Panasonic HDC-SD80		Camcorder

Panasonic NV-GS140		Camcorder

Panasonic NV-GS230		Camcorder

Sony DCR-VX2000		large camcorder

Sony PMW-100			large camcorder

1.6 Props

1.6.1	Fog machines (used in both the small and main stages)

Ground fog machines

Number

Cyro Fog		2

Antari			1

Hazers

Unique		4

MDG			1

Fog machines

Viper 2.6		6

Viper NT		2

Viper S		3

Power Tiny		4

Tiny S			2